# Steps to Setting up Open5GS

Install Ubuntu ver 22.04 in VMWare/Dual Boot

Refer to this 2 link to setup

https://open5gs.org/open5gs/docs/tutorial/01-your-first-lte/

https://nickvsnetworking.com/my-first-5g-core-open5gs-and-ueransim/

# IMPORTANT

Install gcc 11 and g++ 11 first if not the UERANSIM will not work

CC=gcc-11 CXX=g++11 cmake ../

# Setting up MongoDB

sudo apt update

sudo apt install gnupg

curl -fsSL https://pgp.mongodb.com/server-8.0.asc | sudo gpg -o /usr/share/keyrings/mongodb-server-8.0.gpg --dearmor

echo "deb [ arch=amd64,arm64 signed-by=/usr/share/keyrings/mongodb-server-8.0.gpg] https://repo.mongodb.org/apt/ubuntu jammy/mongodb-org/8.0 multiverse" | sudo tee /etc/apt/sources.list.d/mongodb-org-8.0.list

# Install MongoDB

sudo apt update

sudo apt install -y mongodb-org

sudo systemctl start mongod (if '/usr/bin/mongod' is not running)

sudo systemctl enable mongod (ensure to automatically start it on system boot)

# Install Open5GS

sudo add-apt-repository ppa:open5gs/latest

sudo apt update

sudo apt install open5gs

# Install WebUI of Open5GS

# Download and import the Nodesource GPG key

sudo apt update

sudo apt install -y ca-certificates curl gnupg

sudo mkdir -p /etc/apt/keyrings

curl -fsSL https://deb.nodesource.com/gpgkey/nodesource-repo.gpg.key | sudo gpg --dearmor -o /etc/apt/keyrings/nodesource.gpg

# Create deb repository

NODE_MAJOR=20

echo "deb [signed-by=/etc/apt/keyrings/nodesource.gpg] https://deb.nodesource.com/node_$NODE_MAJOR.x nodistro main" | sudo tee /etc/apt/sources.list.d/nodesource.list

# Run Update and Install

sudo apt update

sudo apt install nodejs -y

curl -fsSL https://open5gs.org/open5gs/assets/webui/install | sudo -E bash –

Follow this configuration

/etc/open5gs/nrf.yaml

/etc/open5gs/amf.yaml [Take note of the IP addresses, if running on same device]

/etc/open5gs/upf.yaml

# Restart Open5GS services

sudo systemctl restart open5gs-nrfd

sudo systemctl restart open5gs-amfd

sudo systemctl restart open5gs-upfd

# Register Subscriber to Open5GS

IMSI: 999700000133266

K: 4F068ACCEDC21F2B3E0A4D6D613E1805

OPc: F5FD036A7E0955E199A1CCB3555DD6628

# Setting up UERANSIM

sudo apt update

sudo apt upgrade

sudo apt install make g++ libsctp-dev lksctp-tools

iproute2 sudo snap install cmake –classic

git clone https://github.com/aligungr/UERANSIM

cd UERANSIM

make

make necessary configuration to UERANSIM/config folders

# Steps to start UERANSIM simulation

In the UERANSIM folder:

Start up gNodeB

build/nr-gnb -c config/open5gs-gnb.yaml

Connects UE

Build/nr-ue -c config/open5gs-ue.yaml